QUALITY CONTROL SCHEMES PROCEDURE

1.0 	 Purpose

The purpose of this procedure is to ensure that the test data reported by laboratories are correct and in statistical control, reliable, compared to those generated by other laboratories and above all are fit for its intended use.

2.0	Scope

This procedure covers the quality control schemes and provides minimum general analytical quality control requirements that are appropriate to the test methods used and the type of samples tested. It does not in its present form cover Quality Control procedures for physical tests.

3.0	References:

3.1           AQC.QM-001

3.2	EA-4/10 (formerly EAL-G18), Accreditation for laboratories performing microbiological testing, 1996.

3.3	NATA Tech Note 23 – Guidelines to quality control in Analytical laboratories, 1995.

3.4	ISO 8258, She wart control charts, 1991

3.5	AS 3940, Quality control-Guide to the use of control chart methods, 1990

3.6	ISO/IEC GUIDE 43 part 1” Selection and use of proficiency testing schemes by laboratory accreditation bodies” and part2 ‘development and operation of proficiency testing programmes”, 1995

3.7	NORDTEST Report 452 – A model for Proficiency Testimization, including proficiency testing in the chemical laboratories, 2000

3.8	ILAC – G13 Guidelines for the requirements for the competence of providers of proficiency testing schemes, 2000

4.0	ABBREVIATIONS/TERMS USED

- the definitions in AQC.QM-001 shall apply

- ISO/IEC 17025:2005

- ISO 9001:2015

- ISO/IEC 17000

5.0 RECORDS OF CHANGE

| NO. | Date | Details of changes |  | Title/ Name | Signature |
| --- | --- | --- | --- | --- | --- |
|  | (dd-mm-yyyy) | Page | Clause/Sub-clause and comment | By: |  |
| 01 | 30/06/2021 | All | • Included Tables and Appendices on the Header note. • Changed Version No. from 00 to 01 | QM |  |

6.0 DISTRIBUTION TABLE

The controlled copies of this document are issued in accordance with Appendix C.

| DOC. COPY NO. | DOC. VERSION |  |
| --- | --- | --- |
| 01 | 01 | QUALITY MANAGER |
| 02 | 01 | TECHNICAL MANAGER |
| 03 | 01 | ACCREDITATION BODY (KENAS) |

7.0	Principal Responsibilities

7.1       The analyst or appointed staff is responsible for ensuring that this procedure is followed in the implementation of the quality control measures.

7.2 	Analyst is responsible for the maintaining records of Quality Control activities implemented in the laboratory including records of participation in Proficiency Testing programs.

8.3       The Quality Manager is responsible for including quality control system in the program of quality system audits.

8.0	Procedures

8.1 Internal Quality Control

8.1.1	General Principles

8.1.1.1	The foundation of the Quality Control program of a laboratory is a dual program based on its Internal Quality Control, composed of the daily functions carried out during the collection and analysis of samples to help produce accurate and precise results, and its external Quality Control based on the laboratory performance in proficiency testing programs.

8.1.1.2	The level and type of Quality Control will depend on nature of test, frequency of test, batch size or sample-set size, degree of automation, and test difficulty and reliability.

8.1.1.3	In applying these principles, the laboratory shall consider the following:

a) The sample:  ensure preservation and analysing times of samples are as per procedure to ensure their integrity.

b) Sample preparation procedure: ensure sample preparation procedures are documented and that they include conditioning and cleaning of equipment.

c) Equipment maintenance: Ensure all testing and other laboratory equipment are maintained and are in good working conditions.

d) Equipment calibration: All equipment shall be calibrated regularly as stated in the relevant testing procedure to ensure their constant reliability. In-house calibrations shall also be performed as documented in the test procedure or instructions.

e) Test Methods: All test methods shall be validated and their suitability verified before they are authorized for use in the laboratory.

f) Training of staff: Every member of staff shall be trained to ensure competence in performing the various tests. This shall include training on use, operation and maintenance of the analytical equipment.

g) Reference materials: These shall include the use of CRM, in-house Quality Control sample for method validation, reference calibration standards and assessment of staff competency.

h) Reagents, standards and purity: Reagents and solvent shall be of purity specified in the test methods and blank determinations shall be carried before a new batch of reagents is brought in use. Checks for turbidity and deterioration signs shall be checked regularly to ensure that purity levels are maintained.

i) Records and Reporting: Data records shall be subjected to checking for accuracy and mistakes before they are released for use. Where data exist in electronic form, checking shall still be carried out as required by the procedures and instructions.

8.1.1.4	Based on the above, the analyst shall develop suitable controls to be applied to monitor different types of variation within the test process. However, the sort of Quality Control and that of Proficiency Testing by the laboratory must be sufficient to ensure the validity of test results.

8.1.2 Minimum General Quality Control Requirements

8.1.2.1	The Quality Control procedures applicable to a specific test method or technique will depend on the method, the nature of samples, the analyte of interest and their concentrations. These Quality Control requirements involve:

a) The use of CRM.

b) Quality Control sample.

c) Analysis of spiked samples and determination of recovery at concentrations over range of application of test method.

d) Replicate analyses

e) Blind samples analysis

f) Reanalysis of previous analysed samples

g) Analysis of reagent and laboratory blanks.

h) Analysis of separately sourced and prepared standards.

8.1.2.2 	For effective implementation of the above Quality Control procedures, the laboratory shall define and document the sample-set size or the test batch size for each test covered by the scope of the method.

8.1.2.3 	For each test method, the laboratory shall document the type of Quality Control and check samples required and the frequency with which they should be analysed to ensure that test results are testable.

8.1.2.4 	More specific Quality Control requirements for particular techniques and sample types should be considered.

- 8.1.3 	Evaluation of test results and use of Control charts

8.1.3.1	As shown in Appendix A, different sorts of Quality Control tools may be employed to monitor different types of variation within the test process. Quality Control samples, analysed at intervals in the analytical batch will indicate drift/trends in the system. Use of various types of blanks will also indicate what are the contributions to instrument signal besides those from the analyses; and duplicate tests give a check of repeatability.

8.1.3.2	The reproducibility of the test method results shall be determined by analysis of control samples consisting of reference materials or spiked samples run with every batch of samples over several days or preferably several weeks. The control samples should contain analyte at several concentrations covering the range of application of the test method.

8.1.3.3	The Quality Control samples or in-house reference materials shall be typical samples which over a given period of time are sufficiently stable and homogenous to give the same result and available in sufficient quantities. Over this period monitoring the analysed value of the Quality Control sample shall be monitored to give an indication the random variation in the performance of the test method.

8.1.3.4	The analyst or laboratory shall develop Quality Control samples or source them from the laboratory retained samples.

8.1.3.5	The analyst shall then utilise the Quality Control materials produced to monitor the test method performance by plotting it on a Quality Control chart.

8.2	External Quality Control

8.2.1	General principle

8.2.1.1	A recognized way for the laboratory to monitor its performance against its own requirements and the norm of peer laboratories is through regular participation in Proficiency Testing schemes.

8.2.1.2	Proficiency Test schemes help to highlight reproducibility performance between laboratories and systematic errors i.e. bias. It is also an important means of assessing and demonstrating the reliability of the test data it produces and monitoring traceability at national and international levels.

8.2.1.3 	Before, during and after participating in a Proficiency Test scheme, the analyst shall assess the Proficiency Test scheme based on the above relevant aspects. Including the selection of Proficiency Test and how to deal with its samples, its report and action to be taken on deviating results.

8.2.2	Selection of Proficiency Test schemes

8.2.2.1	The laboratory intending to participate in Proficiency Test scheme shall undertake a search for it and this will be forwarded to the Quality Manager.

8.2.2.2	Information on Proficiency Test organizer and/or available of Proficiency Test-organizers and/or the availability of Proficiency Test can be found from:

a) The Proficiency Test’s contact person

b) Accreditation bodies.

c) Peer laboratories already participated in the Proficiency Test or do know about relevant Proficiency Test’s.

d) A search on the internet, which will probably provide the most actual information. One should include synonymous as “inter-laboratory comparison”,” laboratory evaluation study”, “External quality assessment” etc. use of Proficiency Test

8.2.2.3	Once the information on the Proficiency Test schemes has been obtained, the laboratory/analyst shall compare the situation in the Proficiency Test scheme with the routine situation in laboratory, and to decide which are the most relevant. The analysts can then go ahead and fill the necessary form.

8.2.2.4 	The form, which contains the selection/choice of the laboratory, shall be forwarded to the Quality Manager for approval. The copy of the form shall be maintained by the analyst.

8.2.2.5	If the laboratory is not fully convinced of the relevance of the Proficiency Test’s, the laboratory if possible request the purchase of old samples with Proficiency Test reports and base the selection judgment on this.

8.2.2.6	If the laboratory choice is approved, the laboratory then enrols for participation by forwarding the relevant Proficiency Test scheme details of Proficiency Test contact person including the rounds selected per year.

8.2.2.7	Proficiency Test contact person shall then notify the Proficiency Test-organizer of interest to participate in their scheme, provide registration details, laboratory Proficiency Test requirements and request for any conditions for participation including terms of payment.

8.2.2.8	Once the invoice is received, payment shall be made as per procedure.

8.2.3 	Proficiency Test and distribution of Proficiency Test samples to laboratories

8.2.3.1 	On receipt of Proficiency Test samples, and to ensure that Proficiency Test samples are analysed as part of routine analysis, the Proficiency Test contact person shall register the samples. They are then issued with normal sample identification number.

8.2.3.2 	The sample will then be delivered to the laboratory.

8.2.3.3 	After the laboratory has finished analysing the sample; results will be collected by the analyst and submitted to the Proficiency Test contact for forwarding to the Proficiency Test- organizers.

8.2.4 	Receipt of Proficiency Test Report and Evaluation of performance

8.2.4.1 	The Proficiency Test contact person will usually receive a report on the laboratory’s performance. The Proficiency Test contact person will then forward the report to the QM.

8.2.4.2 	The QM will then forward it to the analyst with comments for action.

8.2.4.3 	The analyst shall then study the report including the comments raised by the QM and compare the laboratory’s present performance with previous record.

8.2.4.4 	In case the participation was unique (no earlier participation) then the analyst shall decide whether or not Corrective Action should be taken.

8.2.4.5 	If yes the analyst shall then recalculate the performance score or use the judgment of the Proficiency Test-organizer. When recalculating the scores, the following actions shall be considered:

a) Use the mean of participants that use the same analytical method

b) Use the claimed or target standard deviation

c) Use the claimed or target value for uncertainty combined with the uncertainty or standard deviation of the assigned value.

d) Correction of the results for recovery

8.2.4.6 	The analyst shall also study the overall performance of all participants in the scheme both satisfactory and unsatisfactory performances.

8.2.5. Proficiency Test Corrective actions

8.2.5.1 	If the analyst or the laboratory finds its performance unsatisfactory, then Corrective Action shall be undertaken. The following list provides guidance on establishing at which point remedial actions need to be taken:

a) After one satisfactory result or z-score are recorded on one round.

b) If at least three unsatisfactory result or z-score are recorded in one round particularly if the tests are linked

c) After at least two consecutive unsatisfactory or questionable results or z-score for any test.

d) If results for any test show a clear trend towards unsatisfactory performance with time over a number of participations.

e) If results for any test over a number of rounds show clear and consistent bias at least one standard deviation above the assigned value, even though results are satisfactory.

8.2.5.2	 If at least any of the above scenarios are identified, the analyst shall document the problem and decide on the Corrective Action necessary. However, before any Corrective Action is taken, the problem shall be analysed thoroughly using the following steps:

a) Analyze the problem based on the result of successive inter-laboratory studies.

b) Internal Quality Control data and record the relevant measurement.

c) Make a plan for Corrective Action.

d) Execute and record the Corrective Action.

e) Check whether the Corrective Action was successful.

9.0	APPENDIX

9.1 Appendix A – Internal Quality Control schedule (spike, repeatability, reproducibility, linearity,

9.2 Appendix B – External quality control schedule (Proficiency Testing Scheme)

9.3 Appendix C - Distribution list of controlled documents.

9.1 APPENDIX A:  – INTERNAL QUALITY CONTROL SCHEDULE

|  | INTERNAL QUALITY CONTROL SCHEDULE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STATISTICAL QUALITY CONTROL TECHNIQUES |  |  |  |  |  |  |  |  |  |  |  |  |
| TEST | JAN | FEB | MRC | APR | MAY | JUN | JLY | AUG | SEP | OCT | NOV |  | DEC |
| EC | CRM | D | S | CRM | B | R | D | CRM | B | R | S |  | CRM |
| TDS | CRM | B | D | CRM | S | R | S | CRM | B | R | D |  | CRM |
| Ph | CRM | B | R | CRM | S | D | B | CRM | S | D | R |  | CRM |
| Phosphorus | CRM | B | R | CRM | D | S | S | CRM | D | R | B |  | CRM |
| Fluoride | CRM | B | S | CRM | R | D | S | CRM | R | B | D |  | CRM |
| Nickel | CRM | D | S | CRM | B | R | D | CRM | B | S | R |  | CRM |
| Cobalt | CRM | R | B | CRM | S | D | R | CRM | S | B | D |  | CRM |
| Chloride | CRM | B | S | CRM | D | R | B | CRM | D | S | R |  | CRM |
| Copper | CRM | D | S | CRM | R | B | B | CRM | R | D | S |  | CRM |
| Hardness | CRM | S | B | CRM | R | S | D | CRM | B | R | D |  | CRM |
| TOC | CRM | D | R | CRM | B | S | D | CRM | S | B | R |  | CRM |
| COD | CRM | R | B | CRM | S | D | R | CRM | S | D | B |  | CRM |
| Coliform&E.coli | CRM | B | D | CRM | R | S | D | CRM | S | R | B |  | CRM |
| Note: - Each parameter should be tabulated and trending after every quality check applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |

NB: R = Repeat, D=Duplicate, S=Spike, B=Blind, CRM= Certified Reference Material

9.2 APPENDIX B: – PROFICIENCY TESTING SCHEME

| PT SCHEME | LOCAL / INTER PT | PT PROVIDER | PT ROUND & YEAR | PT PARAMETERS | No. Of PARTICIPATING LABS |
| --- | --- | --- | --- | --- | --- |

9.3 APPENDIX C:  – DISTRIBUTION LIST OF CONTROLLED DOCUMENTS

| DOC. COPY NO. | DOC. VERSION |  |
| --- | --- | --- |
| 01 | 01 | QUALITY MANAGER |
| 02 | 01 | TECHNICAL MANAGER |
| 03 | 01 | ACCREDITATION BODY (KENAS) |

-End of document-