WASTE DISPOSAL PROCEDURE

1.0 	 Purpose

- This guideline details the procedures to follow in disposing of hazardous waste that is generated in the water testing laboratory in order to minimize risks associated with the disposal of laboratory waste.

2.0	Scope

- This procedure applies to all laboratory personnel within the water testing laboratory who generate and must dispose of hazardous waste. The program is aimed to protect the health and safety of personnel generating or handling hazardous waste, their colleagues, the community and the environment.

3.0   References:

AQC.QM-001

USEPA, May 2005

4.0   ABBREVIATIONS/TERMS USED AND DEFINITIONS

  - For the purpose of this procedure, the definitions in AQC.QM-001 shall apply.

  - A hazardous waste is a solid, liquid, or gaseous material that displays either a “Hazardous Characteristic” or is specifically “listed” by name as a hazardous waste. Characteristic wastes are not listed specifically by their chemical name but they are regulated as hazardous wastes because they exhibit one or more hazardous characteristics. These four characteristics are Ignitability, Corrosivity, Reactivity, and Toxicity.

  - Biological Waste: Biological materials of animal, human, plant or microbial origin.

  - Biohazardous waste: this can include infectious material, contaminated agar plates, live cultures, human cells and blood, and disposables that have been in contact with the above.

  - Broken Glass bin: These are the white plastic bins located within the water testing lab. The contents of these bins are placed into the domestic waste stream, so should not contain any hazardous material.

  - Contaminated Waste bin: This bin contains solid material – glass, plastic, paper which is contaminated with hazardous material

  - Cytotoxic Waste: Any substance contaminated with any residue or preparations that contain materials that are toxic to cells principally by their action on cell reproduction.

  - Domestic glass: Glass not in use in the laboratory such as coffee jars, glass drink bottles.

  - Laboratory glass: Glassware specifically designed or used for laboratory work.

  - Hazardous waste container, liquid: – 20L jerican.

  - Sharps: Objects or devices that have acute, rigid corners, edges, points or protuberances capable of cutting or penetrating the skin e.g. hypodermic needles, glass, scalpel blades and lancets. All sharps are hazardous because of the potential to cause cuts and punctures. Sharps may also be contaminated with toxic, infectious or radioactive materials, which substantially increase the risk potential.

  - Electronic waste:  an item with circuitry or electrical components with power or battery supply that has been assessed as waste.

  - Contaminated solids: Filter paper, Gloves, Tissues, sample vials that is contaminated with hazardous material. This should be placed directly into a contaminated solids bin or bag. These residues are disposed of as hazardous waste.

5.0   RECORDS OF CHANGE

| NO. | Date | Details of changes |  | Title/ Name | Signature |
| --- | --- | --- | --- | --- | --- |
|  | (dd-mm-yyyy) | Page | Clause/Sub-clause and comment | By: |  |
| 01 | 28/07/2021 | All | • Included Tables and Appendices on the Header note. • Changed Version No. from 00 to 01. | QM |  |

6.0   DISTRIBUTION TABLE

The controlled copies of this document are issued in accordance with appendix A.

| DOC. COPY NO. | DOC. VERSION |  |
| --- | --- | --- |
| 01 | 01 | QUALITY MANAGER |
| 02 | 01 | TECHNICAL MANAGER |
| 03 | 01 | ACCREDITATION BODY (KENAS) |

- Principal Responsibilities

  - Lab Manager shall ensure that all workers under his/her direction actively avoid and minimize the generation of hazardous waste. He/she should be able to determine whether a waste is regulated as hazardous and in what hazard classification it falls.

  - The Lab Manager must maintain accurate, up-to-date, and easily retrievable records of sample and waste handling.

  - Technical Manager will be required to establish local procedures that are consistent with these guidelines, and ensure that all staff and students under his direction are familiar with and follow procedures.

  - The Technical Manager must have a Waste Management Plan that is in compliance with the government requirements.

  - The Technician should be familiar with the characteristics of the hazardous waste generated in the laboratory.

  - The Technician will ensure that he/she uses proper personal protection and safety devices at all times.

  - The Technician will take care not to mix incompatible waste. This is a special concern with commingled waste solvents, which must be chemically compatible to ensure that heat generation, gas evolution, or another reaction does not occur.

  - The Technician will use an appropriate container for the collection of liquid waste. Glass bottles are impervious to most chemicals but present a breakage hazard, and narrow-neck bottles are difficult to empty.

  - The Technician shall clearly and securely label waste containers with their contents.

  - The Technician shall place solid chemical waste, such as reaction by-products or contaminated filter paper in an appropriately labelled container. Segregate unwanted reagents for disposal in their original containers, if possible. If original containers are used, labels should be intact and fully legible.

  - The Technician will consider on how to dispose of non-hazardous solid waste in laboratory trash or segregate it for recycling. Check the material safety data sheet, or other appropriate reference to determine toxicity.

8.0 General Principles

8.1 Proper waste categorization can help avoid unnecessary, inappropriate, and costly waste treatment, storage, and disposal. For example, the processes and definitions that the laboratory uses to determine that a waste is hazardous or not will have a great influence on the amount of hazardous waste that the laboratory must manage. Once a waste has been properly categorized, the laboratory can prioritize the waste options for elimination, reduction, or modification.

8.2 Wastes should be minimized where possible. Waste chemicals and solvents are stored in suitable areas whilst awaiting collection and must not be accumulated. Regular disposal from the laboratories must be part of the laboratory waste management plan.

8.3 All waste must be appropriately packaged for the waste category. For example: corrosive waste should be stored in non-metallic containers.

8.4 All liquid waste must be stored in leak proof containers with a screw- top or other secure lid. Snap caps, mis-sized caps and other loose fitting lids are not acceptable.

8.5 Some wastes may be exempt from regulations because of the production process, level of contaminants, volume of waste, or the waste management option chosen.

8.6 The analytical method selected for sample analysis or handling determines the type and volume of waste generated. If two methods will achieve the required measurement quality objectives, the laboratory might select the method that produces the least amount or most easily managed waste. For example, biological laboratories can minimize the use of chemical disinfectants that require handling and disposal as hazardous substances by considering alternative disinfection procedures, such as autoclaving, that do not generate hazardous waste.

8.7 Untrained staff and students are not to handle hazardous wastes and must not be given responsibility for them.

8.8 Personal Protective Equipment should be a consideration when handling chemical waste.

8.9Reference should be made to the Material Safety Data Sheet at all times.

8.10 Amounts of reagents and materials purchased by a laboratory often are determined by price discounts available on large quantities, instead of the amount required. The real cost of purchasing these materials should include the initial purchase price plus disposal costs (lifetime costs). Procurement of hazardous material should be initiated only if a nonhazardous substitute is not available. Rotating chemical stock (first in, first out) also can help avoid expiration.

9.0 Laboratory Procedures

9.1 The Lab Manager must inspect areas in which waste containers are stored, at least weekly, looking for leaks and deterioration caused by corrosion or other factors.

9.2 All sample and sample preparation (extraction) wastes should be segregated according to chemical class. Acidic wastes should be stored separately from basic wastes, oxidizers should be stored separately from organics, and cyanide positive wastes should be stored separately from acids to reduce the possibility of inadvertent potentially dangerous releases/exposures, fires, or increased hazardous status.

9.3 Each type of waste (based on the specific chemical contamination, level, and initial generation site) must be stored in a separate, labeled container to help reduce multi-hazardous wastes and assist in source reduction. The label must contain the chemical contaminant, the waste generation point, the date of generation, and the initials of the technician responsible for the initial generation of the waste where applicable.

9.4 The laboratory must use proper storage containers, limiting the container size to properly reflect the amount of sample and sample waste expected to be generated. The storage container must be properly designed to double as the correct transport/disposal container (e.g., drum, jericans) to reduce handling of the waste materials. Storage containers must be made of proper materials with sealable lids.

9.5 Solid wastes should be collected and segregated. Most solid wastes cannot be treated at the laboratory and should be properly characterized and disposed of at an approved disposal facility.

9.6 Empty containers that once held hazardous materials are not regulated as hazardous waste if they meet the definition of “empty.” Empty means that all hazardous materials have been removed from the container. Containers that held acutely hazardous waste are considered empty only after being triple rinsed with a solvent capable of removing the acutely hazardous waste residue. Disposable laboratory equipment also can be decontaminated by triple rinsing with a solvent capable of removing residue. The solvent rinse then must be managed as acutely hazardous waste.

9.7 Laboratory glassware and other re-useable equipment can be decontaminated using the same procedures used to decontaminate empty containers, and cleaned in the same manner used to clean glassware for laboratory use. Cleaning solvents and procedures for these materials are somewhat dependent on the intended use.

9.8 Prior to disposal or transfer of waste to a waste hauler, the water testing laboratory will identify whether the waste is (1) a solid waste and/or (2) a hazardous waste.

9.9 The laboratory should use either documented analytical results or manufacturer’s certifications of reagents to determine the hazard characteristics of samples and related waste.

9.10 The selection of a proper waste hauler is important. The laboratory is responsible for waste shipments and should expect proper documentation from the transporter for all shipments.

9.11 Waste is removed to a certified incinerator (typically in bulk shipping containers) and destroyed by burning. Each incinerator operator is certified to accept specific wastes in composition, quantity, and concentration. The laboratory will need to ensure proper disposal can be met and that the shipped material meets the requirements of the incinerator permit.

10. References

10.1 Working with contaminated Hazardous Waste index

http://staff.uow.edu.au/ohs/workingsafely/hazardouswaste/

10.2 A step-by-step guide to contaminated Waste Disposal Guidelines

http://staff.uow.edu.au/content/groups/public/@web/@ohs/documents/doc/uow017033.pdf

10.3 Contaminated Waste Disposal Guidelines

http://staff.uow.edu.au/content/groups/public/@web/@ohs/documents/doc/uow017032.pdf

10.4 Armour, M.A. 2003. Hazardous Laboratory Chemicals Disposal Guide, 3rd Edition. Boca Raton, FL: CRC Press.

10.5 Keegan, R.J. IV, and R.J. Keegan. 1997. Hazardous Materials, Substances and Wastes Compliance Guide. Kutztown, PA: Hazardous Materials Publishing Co.

10.6 James Lieberman. 1994. A Practical Guide for Hazardous Waste Management, Administration and Compliance. ISBN: 9781566701150.

10.7 U.S. Environmental Protection Agency. May 2005. Handbook on the Management of Munitions Response Action. Interim Final. EPA 505 B-01-001. http://epa.gov/fedfac/pdf/mra_hbook_5_05.pdf (accessed 11/24/09).

11.0 APPENDIX A:  – DISTRIBUTION LIST OF CONTROLLED DOCUMENTS

| DOC. COPY NO. | DOC. VERSION |  |
| --- | --- | --- |
| 01 | 01 | QUALITY MANAGER |
| 02 | 01 | TECHNICAL MANAGER |
| 03 | 01 | ACCREDITATION BODY (KENAS) |

-End of document-