SAMPLE COLLECTION, HANDLING AND STORAGE PROCEDURE

1.0   PURPOSE

The purpose of this document is to define the standard operating procedure (SOP) for sample management for the Aquacheck Laboratories limited which includes; sample collection, handling, numbering, transportation, storage and documentation for samples

Collected for chemical and other analyses for effective Sample Management

This procedure is to be used together with other SOPs;

- To achieve appropriate sample collection, handling and transportation of samples to the laboratory which is critical to the outcome of the analysis of any water sample.

- to provide and maintain consistent standard procedures to label, transport, ship, and maintain Chain of Custody for Aquacheck Laboratories limited

2.0   SCOPE

- This procedure acts as a guide to all laboratory personnel during water sampling, handling and transportation to the testing laboratory. This guide is therefore established to specify how to collect and preserve samples to ensure the most valid and accurate data is produced.

3.0   RESPONSIBILITIES AND QUALIFICATIONS

The Quality Manager and the laboratory personnel will have the overall responsibility for implementing this SOP. They will be responsible for assigning appropriate staff to implement this SOP and for ensuring that the procedures are followed accurately.

All personnel are required to have a complete understanding of the procedures described within this SOP and to receive specific training regarding these procedures if necessary.

All environmental staff and laboratory staff are responsible for reporting deviations from this SOP to the Project Manager

- SAMPLE COLLECTION

4.1 Sampling for Microbiology

- Use a clean, dry, leak-proof, sterile sample container capable of holding at least 1L of water.

- Wear gloves when collecting samples .Samples must be taken aseptically to prevent contamination. Collect a minimum of 1L of water.

- Chlorinated water samples must be collected in sterile sample bottles with sodium thiosulphate, in order to neutralise residual chlorine or other halogens.

- Keep the lid on the sample bottle until you are ready to take the sample and during sampling, the container lid should not come in contact with anything that will contaminate the sample.

- The sample should be taken without rinsing the container, and the lid replaced immediately.

- Do not fill the container leave at least 2.5 cm air space to allow mixing of the sample at the laboratory.

- When sampling from a tap, it should be clean, free of attachments and in good repair. Disinfect the tap (refer to page 6) and let it run to waste for 2-3 minutes. Reduce the water flow to prevent splashing and collect the sample.

- After sampling, label the sample and fill the sampling form, chain of custody form or other relevant documents.

- Chill samples to 1 – 10 °C as soon as possible by storing in a refrigerator or by pre-icing them in a cooler prior to shipment.

- All microbiology samples should reach the lab within 24 hours of sample collection to allow analysis to begin.

NB: Use one of the following three methods:

- Swab or spray the nozzle and part of the body liberally with 100% absolute alcohol or methylated spirits and flame it. Open the tap fully and let the water run to waste for 2-3 minutes, or for a time sufficient to permit clearing of the service line. The tap should be sufficiently cooled before taking the sample.

- Swab or spray the inside and outside of the tap thoroughly with hypochlorite solution (bleach) made up freshly to 1% available chlorine. Leave for 2-3 minutes to allow disinfection to take place. Run the water to waste for a sufficient period to ensure that all hypochlorite solution has been removed before collecting the sample.

- Using a butane or propane burner, disinfect the tap by flaming, starting at the nozzle and working back to the body of the tap until the water in the nozzle boils. After flaming, water from the tap should be run to waste until cool before a sample is taken. Disinfection by flaming is suitable for metal taps without plastic fittings or other heat-sensitive components.

4.2 Sampling for Chemistry

- Use a clean high-density polyethylene screw-capped bottles, of approximately 1L in volume.

- Rinse the sample containers twice with the water to be collected before taking the samples.

- The 1 L bottle should be filled to the brim, to prevent oxidation of analytes.

- After sampling, label the sample and fill the sampling form, chain of custody form or other relevant documents.

NB: water samples collected in plastic or glass bottles for trace metals analysis will be added acid at the laboratory to allow recovery of metals adsorbed to the container. This acid is also used for perseveration of the sample at Aquacheck laboratories whenever necessary.

4.3 Sampling point guidelines

4.3.1 Wells, Boreholes

- Disinfect the taps using one of the methods described on section 4.1. above

- Mechanical pumps should be run for at least 2-3 minutes and at least 5 minutes when using hand pump before sampling.

- Follow the procedure of sampling for Chemistry/Microbiology depending on the tests required.

4.3.2 Rivers, Lakes and Reservoirs

- Collect the sample by holding the bottle near its base and plunging it, neck downward, below the surface.

- Turn the bottle until the neck points slightly upward and the mouth is directed toward the current.

- For a reservoir/dam, create a current artificially by pushing the bottle forward horizontally in a direction away from the hand.

- The sample should be collected from the middle part of the dam as much as possible and not from the periphery.

- Follow the procedure of sampling for Chemistry/Microbiology depending on the tests required.

5.0 EQUIPMENT AND MATERIALS

The following Materials and equipment will be required during sampling:

• Sample containers (collection bottles for water samples)

• whirl bags or other sample bags

• Ice packs

• Tape (if necessary)

• Cooler Box

• Waterproof Pens

• Chain of Custody (COC) Forms

• Weatherproof Sample Labels

• Appropriate database forms (if necessary)

• Gloves

• Packing material

6.0 SAMPLE HANDLING

- Use proper sample containers, preservatives where necessary, and transportation procedures

6.1 Sample Preservation

- All water samples will be stored on frozen ice packs in an insulated cooler to obtain a temperature of 4ºC immediately after sample collection.

- Trace element sample bottles will be preserved with ultrapure nitric acid to a pH less than 2 at least 16 hours before analysis and will be clearly labeled “w/ HNO3”.

7.0 SAMPLE TRANSPORTATION

- After sampling, water samples will then be placed in a cooler box with frozen icepacks for transportation to the laboratory.

- Most samples will be transported to the laboratory by courier services or PSV (public service vehicle) Parcel services and receipts should be retained for documentation and sample tracking.

- Samples collected in glass containers will be packed in foam liners, bubble packing or Styrofoam to ensure that no breakage occurs during transport.

- A completed chain-of-custody (COC) form (Appendix 1) for each sample container (cooler box) will be taped to the inside of the cooler lid.

- Wrap each cooler with strapping tape in two places to secure the lids.

APPENDIX 1: CHAIN OF CUSTODY FORM

AQUACHECK LABORATORIES LIMITED

P.O. Box 12084 - 00400

Westlands Commercial Centre

Ring Road,Nairobi kenya

Email: aquachecklab@gmail.com

TEL: 0720375629/0721 800 041

AQC.QP5-F27: CHAIN OF CUSTODY FORM (COC)

Field COC form

| Chain of custody record |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Preservative: |  | Type of test: Full Chemical analysis □ Microbiological analysis □ Other…………………………………………………………………………….. |  | Client: Contact: Adress: |  |
| Number of Containers: |  |  |  |  |  |
| Field sampler: Sampling form No: Signature: |  |  |  |  |  |
| Date | Time | Site/Location | Sample type | Sample ID | Remarks |
| Relinquished by: |  | Received by: |  | Comments |  |
| Signature: |  | Signature: |  |  |  |

Laboratory COC form

| Chain of custody record |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Client name: Client contact: |  | Sampling date: Sampling location: |  | Sampling form No: Test Requisition form No: |  |
| Sample description: Test requested:…………………………………………………………………………………………………………………………….. |  |  |  |  |  |
| Sample ID | Number of containers | Date received | Date of analysis | Date of Report issue | Comments |
| Sample ID | Relinquished by: Signature: | Received by: Signature: | Time | Date | Reason for change of custody |
| Sample ID | Number of containers | Date stored | Storage period | Disposal date | Comments |
| Sample ID | Relinquished by: Signature: | Disposed by: Signature: | Time | Date | Reason for change of custody |

APPENDIX II:SAMPLING FORM

AQUACHECK LABORATORIES LIMITED

P.O. Box 12084 - 00400

Westlands Commercial Centre

Ring Road,Nairobi kenya

Email: aquachecklab@gmail.com

TEL: 0720375629/0721 800 041

AQC.QP5-F28: SAMPLING FORM

| Sample ID: |  | Sample type: |
| --- | --- | --- |
| Sampling |  | Comments |
| Date and time |  |  |
| Site/Location |  |  |
| Approx.sampling depth (distance from surface): |  |  |
| Sample volume: |  |  |
| Water temperature: |  |  |
| Transport temperature (after sampling): |  |  |
| Drilling date (for wells/boreholes only) |  |  |
| Special observations/deviation from sampling procedures |  |  |
| Storage |  | Comments |
| Storage time: |  |  |
| Storage temperature: |  |  |
| Special observations: |  |  |
| Transportation |  | Comments |
| Date sample sent: |  |  |
| Date sample received: |  |  |
| Condition upon receipt: |  |  |
| Special observations: |  |  |
| Sampled by: | Signature: |  |

APPENDIX III: REFERENCES

1.USEPA 1996, Method 1669, Sampling Ambient Water for Trace Metals at EPA Water Quality Criteria Levels, United States Environmental Protection Agency, Washington, viewed 30 August 2016, www3.epa.gov/caddis/pdf/Metals Sampling EPA method 1669.pdf.